Functions required for a guitar technician / Luthier user

- User registration - issuing a unique identification.

- Signing an undertaking to act according to the RiffTales guidelines.

- Business registration:

- Contact

- Guitar set up and repair price list

- Shop (sell)

- Photos

- Serial number identification

Setup table with verified adjustments:	Comments:

| Truss Rod Adjustment: This ensures the neck is straight and has the right amount of relief. |  |
| --- | --- |
| String Replacement: Customer’s preferred brand and gauge / Technician/Luthier’s recommendation. |  |
| Action Adjustment: adjusting the saddle height at the bridge (and sometimes the nut) to ensure the strings are at a comfortable height. |  |
| Intonation Setting: adjusting the saddle positions (on electric guitars) or the bridge (on acoustic guitars) to achieve correct intonation. |  |
| Nut Inspection/Adjustment: If the slots in the nut are too deep, too wide, or too tight, it can cause tuning instability and string buzz. The luthier may file and adjust these slots or recommend replacing the nut. |  |
| Cleaning: cleaning the fretboard, body, headstock, and any hardware. It might also involve oiling the fretboard if it's particularly dry. |  |
| Fret Inspection/Polishing: checking for worn or uneven frets. A full fret job might not be included in a standard setup |  |
| Electronics Check (for electric guitars): checking the guitar's electronics for any issues, cleaning potentiometers if they're scratchy, ensuring the jack is secure, and checking that all pickups are working correctly. |  |
| Hardware Tightening: tuning machines, strap buttons, and any screws, will be checked and tightened if needed. |  |
| Feedback/Recommendations: potential problems or suggest further repairs or modifications that could benefit the guitar. |  |

Modifications table with verified adjustments:

Changes made to the guitar, dated and signed by technicians (identified)

Uploading images \ selecting stock images of the changes

Here's a list of potential modifications one might consider for a guitar:

Cosmetic Modifications:

Comments:			Price:

| Pickguard Change: Swapping out for a different color or design. |  |  |
| --- | --- | --- |
| Knob Replacement: Changing volume and tone knobs to different styles or materials. |  |  |
| Inlay Work: Adding decorative inlays to the fretboard or body. |  |  |
| Finish Work: Refinishing the guitar in a different color or with a unique design. |  |  |
| Headstock Logo: Adding or changing the logo or design. |  |  |

Functional and Hardware Modifications:

Comments:			Price:

| Tuning Machine Upgrade: Replacing with locking tuners or higher quality machines. |  |  |
| --- | --- | --- |
| Bridge and Saddle Upgrade: Changing to better materials or a design that offers better intonation or sustain. |  |  |
| Nut Replacement: Using bone, Tusq, or other superior materials for improved tone and sustain. |  |  |
| Fret Replacement: Upgrading to jumbo frets or using a different material. |  |  |
| Strap Locks: Installing locks to prevent the strap from accidentally detaching. |  |  |

Electronics and Sound Modifications (Primarily for Electric Guitars):

Comments:			Price:

| Pickup Replacement: Swapping pickups for a different tone, output, or brand. |  |  |
| --- | --- | --- |
| Potentiometer Upgrade: Changing to pots with different resistances or better quality. |  |  |
| Switch Upgrade: Using a different switch type, like a 5-way or a push-pull pot. |  |  |
| Capacitor Change: Using a different value or type for tone controls. |  |  |
| Wiring Modifications: Different wiring schemes like coil splitting, series/parallel wiring, or phase reversal. |  |  |
| Active Electronics: Adding a preamp or active pickups that require a battery. |  |  |
| Tone-Shaping Circuits: Installing mid-boost circuits, treble bleed circuits, etc. |  |  |
| Kill Switch Installation: A button that mutes the guitar when pressed. |  |  |
| Output Jack Replacement: Upgrading to a more durable jack or a stereo jack for certain applications. |  |  |

Playability Modifications: 			Comments:		Price:

| Scalloped Fretboard: Removing wood between the frets for a unique feel. |  |  |
| --- | --- | --- |
| Compound Radius Fretboard: Changing the fretboard to have a more rounded radius at one end and a flatter one at the other. |  |  |
| Neck Profile Change: Shaping the back of the neck to a different profile, e.g., from a "C" shape to a "D" shape. |  |  |

Acoustic Guitar-Specific Modifications: Comments:		Price:

| Pickup Installation: Adding an acoustic pickup system. |  |  |
| --- | --- | --- |
| Bridge Pin Upgrade: Changing to bone or Tusq bridge pins for improved tone. |  |  |
| Soundhole Cover: Adding a removable cover to reduce feedback. |  |  |

Other Modifications:			   Comments:    		Price:

| Tremolo/ Vibrato System Upgrade: Installing or changing to a different system, like a Floyd Rose or Bigsby. |  |  |
| --- | --- | --- |
| Extended Range: Modifying or replacing the neck to add more frets or strings. |  |  |
| Synth/MIDI Access: Adding components to allow the guitar to interface with synthesizers or MIDI equipment. |  |  |